PARAMETERS MONITORED BY STATE THAT ARE NOT FEDERALLY REGULATED:

| PARAMETERS | SYSTEM TYPE | MAXIMUM CONTAMINANT LEVEL |
| --- | --- | --- |
| TOTAL ALKALINITY* | COMM & NTNC | NOT APPLICABLE |
| TOTAL HARDNESS | “ “ | “ “ |
| LOSS ON IGNITION | “ “ | “ “ |
| POTASSIUM | “ “ | “ “ |
| CARBONATE ALKALINITY* | “ “ | “ “ |
| CALCIUM HARDNESS* | “ “ | “ “ |
| SODIUM | “ “ | “ “ |
| FREE CARBON DIOXIDE | “ “ | “ “ |
| BICARBONATE ALKALINITY* | “ “ | “ “ |
| MAGNESIUM HARDNESS | “ “ | “ “ |

NATIONAL DRINKING WATER REGULATIONS:

# SECONDARY CONTAMINANT

| CONTAMINANT | SYSTEM TYPE | MAXIMUM CONTAMINANT LEVEL |
| --- | --- | --- |
| ALUMINUM (2) | COMM & NTNC | 0.05 TO 0.2mg |
| CHLORIDE (1) | “ “ | 250 MG/L |
| COLOR (1) | “ “ | 15 COLOR UNITS |
| CORROSIVITY (1) | “ “ | NONCORROSIVE |
| FOAMING AGENTS (1) | “ “ | 0.5 MG/L |
| FLUORIDE (1) | “ “ | 2.0 MG/L |
| IRON (1) | “ “ | 0.3 MG/L |
| MANGANESE (1) | “ “ | 0.05 MG/L |

NATIONAL DRINKING WATER REGULATIONS:

SECONDARY CONTAMINANT (CONTINUED)

| CONTAMINANT | SYSTEM TYPE | MAXIMUM CONTAMINANT LEVEL |
| --- | --- | --- |
| ODOR (1) | COMM & NTNC | 3 THRESHOLD ODOR NO. |
| pH (1) | “ “ | 6.5 – 8.5 |
| SILVER (2) | “ “ | 0.1 MG/L |
| SULFATE (1) | “ “ | 250 MG/L |
| TOTAL DISSOLVED (1) SOLIDS (TDS) | “ “ | 500 MG/L |
| ZINC (1) | “ “ | 5 MG/L |

*THESE CONSTITUENTS ARE ALSO MONITORED AS WATER QUALITY PARAMETERS UNDER THE LEAD AND COPPER RULE.  SEE THE MONITORING REQUIREMENTS FOR LEAD AND COPPER.

NON-FEDERALLY REGULATED PARAMETERS AND SECONDARY CONTAMINANTS:

MONITORING:

GROUND WATER SYSTEMS: - SAMPLED EVERY THREE YEARS.

SURFACE WATER SYSTEMS: - SAMPLED ONCE A YEAR.

CHECK SAMPLING & PUBLIC NOTIFICATION:

NOT REQUIRED WITH THE EXCEPTION OF SODIUM AND FLUORIDE.  SODIUM LEVELS ARE SENT TO LOCAL PHU BY DHH.  COMMUNITY SYSTEMS REQUIRED TO MAKE PUBLIC NOTICE IF FLUORIDE IS BETWEEN THE SECONDARY MCL OF 2.0 MG/L AND THE PRIMARY MCL OF 4.0 MG/L.

NATIONAL PRIMARY DRINKING WATER REGULATIONS:

MICROBIOLOGICAL (COLIFORM)

MONITORING:

ALL COMMUNITY, NON-COMMUNITY USING GROUNDWATER AND SERVING MORE THAN 1000 PERSONS, AND ANY NON-COMMUNITY USING ANY SURFACE WATER, OR USING GROUNDWATER UNDER THE DIRECT INFLUENCE OF SURFACE WATER MUST MONITOR IN ACCORDANCE WITH TABLE 1.  NON-COMMUNITY USING GROUNDWATER AND SERVING LESS THAN 1000 PERSONS, MUST ROUTINELY MONITOR ONCE EACH QUARTER.

TABLE 1

TOTAL COLIFORM SAMPLING REQUIREMENTS

| POPULATION SERVED | # OF SAMPLES/MO | POPULATION SERVED | # OF SAMPLES/MO |
| --- | --- | --- | --- |
| 25 TO 1,000 | 1 | 59,001 TO 70,000 | 70 |
| 1,001 TO 2,500 | 2 | 70,001 TO 83,000 | 80 |
| 2,501 TO 3,300 | 3 | 83,001 TO 96,000 | 90 |
| 3,301 TO 4,100 | 4 | 96,001 TO 130,000 | 100 |
| 4,101 TO 4,900 | 5 | 130,001 TO 220,000 | 120 |
| 4,901 TO 5,800 | 6 | 220,001 TO 320,000 | 150 |
| 5,801 TO 6,700 | 7 | 320,001 TO 450,000 | 180 |
| 6,701 TO 7,600 | 8 | 450,001 TO 600,000 | 210 |
| 7,601 TO 8,500 | 9 | 600,001 TO 780,000 | 240 |
| 8,501 TO 12,900 | 10 | 780,001 TO 970,000 | 270 |
| 12,901 TO 17,200 | 15 | 970,001 TO 1,230,000 | 300 |
| 17,201 TO 21,500 | 20 | 1,230,001 TO 1,520,000 | 330 |
| 21,501 TO 25,000 | 25 | 1,520,001 TO 1,850,000 | 360 |
| 25,001 TO 33,000 | 30 | 1,850,001 TO 2,270,000 | 390 |
| 33,001 TO 41,000 | 40 | 2,270,001 TO 3,020,000 | 420 |
| 41,001 TO 50,000 | 50 | 3,020,001 TO 3,960,000 | 450 |
| 50,001 TO 59,000 | 60 | 3,960,001 OR MORE | 480 |

NATIONAL PRIMARY DRINKING WATER REGULATIONS:

MICROBIOLOGICAL (COLIFORM)

MAXIMUM CONTAMINANT LEVEL:

- THE MAXIMUM CONTAMINANT LEVEL (MCL) IS BASED ON THE PRESENCE OR ABSENCE OF TOTAL COLIFORM RATHER THAN ON COLIFORM DENSITY.

- IF 40 OR MORE DISTRIBUTION SAMPLES ARE COLLECTED PER MONTH, NO MORE THAN 5% OF THE MONTHLY SAMPLES MAY BE TOTAL COLIFORM POSITIVE.

- IF LESS THAN 40 DISTRIBUTION SYSTEM SAMPLES ARE COLLECTED PER MONTH, NO MORE THAN ONE SAMPLE PER MONTH MAY BE TOTAL COLIFORM POSITIVE.

- AN ACUTE VIOLATION OCCURS WHEN:

- A COLIFORM-POSITIVE ORIGINAL SAMPLE THAT IS ALSO POSITIVE FOR FECAL COLIFORM (OR E. COLI) IS FOLLOWED BY A POSITIVE COLIFORM REPEAT SAMPLE

OR

- A COLIFORM-POSITIVE ORIGINAL SAMPLE IS FOLLOWED BY A COLIFORM-POSITIVE REPEAT SAMPLE THAT IS ALSO POSITIVE FOR FECAL COLIFORM (OR E. COLI).

THE NUMBER OF REPEAT SAMPLES SHALL BE IN ACCORDANCE WITH TABLE 2.

TABLE 2

MONITORING AND REPEAT SAMPLE FREQUENCY

AFTER A TOTAL COLIFORM POSITIVE ROUTINE SAMPLE

| # OF ROUTINE SAMPLES/MONTH | # OF REPEAT SAMPLES/POSITIVE | # OF ROUTINE SAMPLES NEXT MONTH |
| --- | --- | --- |
| 1 OR FEWER/MONTH | 4 | 5/MONTH |
| 2/MONTH | 3 | 5/MONTH |
| 3/MONTH | 3 | 5/MONTH |
| 4/MONTH | 3 | 5/MONTH |
| 5 OR MORE/MONTH | 3 | TABLE 1 |

PUBLIC NOTIFICATION:

IF THE MCL IS EXCEEDED, THE SUPPLIER OF WATER IS REQUIRED TO PROVIDE PUBLIC NOTICE IN A DAILY OR WEEKLY NEWSPAPER WITHIN 14 DAYS OF BEING NOTIFIED BY THE OFFICE OF PUBLIC HEALTH.  IN ADDITION TO NEWSPAPER NOTICE, A DIRECT MAIL OR HAND DELIVERED NOTICE SHALL BE ISSUED WITHIN 45 DAYS.  FOR AN ACUTE MCL VIOLATION, A NOTICE SHALL ALSO BE FURNISHED, BY COMMUNITY SYSTEMS ONLY, TO RADIO AND TV STATIONS SERVING THE AREA WITHIN 72 HOURS.

| National Primary Drinking Water Standards |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MCLG | MCL | Potential Health Effects from |  |  |  |
| Contaminants | (mg/L) | (mg/L) | Ingestion of Water |  | Sources of Contaminant in Drinking Water |  |
| Fluoride | 4.0 | 4.0 | Skeletal and dental fluorosis |  | Natural deposits; fertilizer, aluminum industries; water additive |  |
| Volatile Organics |  |  |  |  |  |  |
| Benzene | zero | 0.005 | Cancer |  | Some foods; gas, drugs, pesticide, paint, plastic industries |  |
| Carbon Tetrachloride | zero | 0.005 | Cancer |  | Solvents and their degradation products |  |
| p-Dichlorobenzene | 0.075 | 0.075 | Cancer |  | Room and water deodorants, and “mothballs” |  |
| 1,2-Dichloroethane | zero | 0.005 | Cancer |  | Leaded gasoline, fumigants, paints |  |
| 1,1-Dichloroethylene | 0.007 | 0.007 | Cancer |  | Plastics, dyes, perfumes, paints |  |
| Trichloroethylene | zero | 0.005 | Cancer |  | Textiles, adhesives and metal degreasers |  |
| 1,1,1-Trichloroethane | 0.2 | 0.2 | Liver Nervous system effects |  | Adhesives, aerosols, textiles, paints, inks, metal degreasers |  |
| Vinyl chloride | zero | 0.002 | Cancer |  | May leach from PVC pipe; formed by solvent break down |  |
| Coliform and surface water Treatment |  |  |  |  |  |  |
| Giardia Lamblia | zero | TT | Gastroenteric disease |  | Human and animal fecal waste |  |
| Legionella | N/A | TT | Legionnaire’s disease |  | Indigenous to natural water; can grow in water heating systems |  |
| Standard Plate count | N/A | TT | indicates water quality, effectiveness of treatment |  |  |  |
| Total coliform | zero | <5%+ | Indicates gastroenteric pathogens |  | Human and animal fecal waste |  |
| Turbidity | N/A | TT | Interferes with disinfection filtration |  | Soil runoff |  |
| Viruses | zero | TT | Gastroenteric disease |  | Human and animal fecal waste |  |
| Inorganics |  |  |  |  |  |  |
| Antimony | zero | 0.006 | Cancer |  | Fire retardants, ceramics, electronics, fireworks, solder |  |
| Asbestos (>10um) | 7MFL | 7MFL | Cancer |  | Natural deposits; asbestos cement in water systems |  |
| Barium | 2 | 2 | Circulatory system effects |  | Natural deposits; pigments, epoxy sealants, spent coal. |  |
| Beryllium | 0.004 | 0.004 | Bone, lung damage |  | Electrical, aerospace, defense industries |  |
| Cadmium | 0.005 | 0.005 | Kidney effects |  | Galvanized pipe corrosion; natural deposits; batteries, paints |  |
| Chromium (total) | 0.1 | 0.1 | Liver, kidney, circulatory disorders |  | Natural deposits; mining, electroplating, pigments. |  |
| Cyanide | 0.2 | 0.2 | Thyroid, nervous system damage |  | Electroplating, steel, plastics, mining, fertilizer |  |
| Mercury (inorganic) | 0.002 | 0.002 | Kidney, nervous system disorders |  | Crop runoff; natural deposits; batteries, electrical switches |  |
| Nitrate | 10 | 10 | Methemoglobulinemia |  | Animal waste, fertilizer, natural deposits, septic tanks, sewage |  |
| Nitrite | 1 | 1 | Methemoglobulinemia |  | Same as nitrate; rapidly converted to nitrate |  |
| Selenium | 0.05 | 0.05 | Liver damage |  | natural deposits; mining, smelting, coal/oil combustion |  |
| Thallium | 0.0005 | 0.002 | Kidney, liver, brain, intestinal |  | Electronics, drugs, alloys, glass |  |
| Organics |  |  |  |  |  |  |
| Acrylamide | zero | TT | Cancer, nervous system effects |  | Polymers used in sewage/wastewater treatment |  |
| Adipate, (di (2-ethylhexyl) | 0.4 | 0.4 | Decreased body weight |  | Synthetic rubber, food packaging, cosmetics |  |
| Alachlor | zero | 0.002 | Cancer |  | Runoff from herbicide on corn, soybeans, other crops |  |
| Atrazine | 0.003 | 0.003 | Mammary gland tumors |  | Runoff from use as herbicide on corn and non-cropland |  |
| Carbofuran | 0.04 | 0.04 | Nervous, reproductive system effects |  | Soil fumigant on corn and cotton; restricted in some areas |  |
| Chlordane | zero | 0.002 | Cancer |  | Leaching from soil treatment for termites |  |
| Chlorobenzene | 0.1 | 0.1 | Nervous system and liver effects |  | Waste solvent from metal degreasing processes |  |
| Dalapon | 0.2 | 0.2 | Liver and kidney effects |  | Herbicide on orchards, beans, coffee, lawns, road/railways |  |
| Dibromochloropropane | zero | 0.0002 | Cancer |  | Soil fumigant on soybeans, cotton, pineapple, orchards |  |
| o-Dichlorobenzene | 0.6 | 0.6 | Liver, kidney, blood cell damage |  | Paints, engine cleaning compounds, dyes, chemical wastes |  |
| cis-1,2-Dichloroethylene | 0.07 | 0.07 | Liver, kidney, nervous, circulatory |  | Waste industrial extraction solvents |  |
| Notes: *Contaminants with interim standards which have been revised. TT= Special treatment techniques required MFL= million fibers per liter. += less than 5% positive samples |  |  |  |  |  |  |
|  | MCLG | MCL |  | Potential Health Effects from |  |  |
| Contaminants | (mg/L) | (mg/L) |  | Ingestion of Water |  | Sources of Contaminant in Drinking Water |
| Organics (continued) |  |  |  |  |  |  |
| trans-1,2- Dichloroethylene | 0.1 | 0.1 |  | Liver, kidney, nervous, circulatory |  | Waste industrial extraction solvents |
| Dichloromethane | zero | 0.005 |  | Cancer |  | Paint stripper, metal degreaser, propellant, extraction |
| 1,2- Dichloropropane | zero | 0.005 |  | Liver, kidney effects; Cancer |  | Soil fumigant; waste industrial solvents |
| Dinoseb | 0.007 | 0.007 |  | Thyroid, reproductive organ damage |  | Runoff of herbicide from crop and non-crop applications |
| Dioxin | zero | 0.00000003 |  | Cancer |  | Chemical production by-product; impurity in herbicides |
| Diquat | 0.02 | 0.02 |  | Liver kidney, eye effects |  | Runoff of herbicide on land & aquatic weeds |
| 2,4- D* | 0.07 | 0.07 |  | Liver and kidney damage |  | Runoff from herbicide on wheat, corn, rangelands, lawns |
| Endothall | 0.1 | 0.1 |  | Liver, kidney, gastrointestinal |  | Herbicide on crops, land/aquatic weeds; rapidly degraded |
| Endrin | 0.002 | 0.002 |  | Liver, kidney, heart damage |  | Pesticide on insects, rodents, birds; restricted since 1980 |
| Epichlorohydrin | zero | TT |  | Cancer |  | Water treatment chemicals; waste epoxy resins, coatings |
| Ethylbenzene | 0.7 | 0.7 |  | Liver, kidney, nervous system |  | Gasoline; insecticides; chemical manufacturing wastes |
| Ethylene dibromide | zero | 0.00005 |  | Cancer |  | Leaded gasoline additives; leaching of soil fumigant |
| Glyphosate | 0.7 | 0.7 |  | Liver, kidney damage |  | Herbicide on grasses, weeds, brush |
| Heptachlor | zero | 0.0004 |  | Cancer |  | Leaching of insecticide for termites, very few crops |
| Heptachlor epoxide | zero | 0.002 |  | Cancer |  | Biodegradation of heptachlor |
| Hexachlorobenzene | zero | 0.001 |  | Cancer |  | Pesticide production waste by-product |
| Hexachlorocyclopentadiene | 0.05 | 0.05 |  | Kidney, stomach damage |  | Pesticide production intermediate |
| Lindane | 0.0002 | 0.0002 |  | Liver, kidney, nerve, immune, circul. |  | Insecticide on cattle, lumber, gardens; restricted 1983 |
| Methoxychlor | 0.04 | 0.04 |  | Growth, liver, kidney, nerve effects |  | Insecticide for fruits, vegetables, alfalfa, livestock, pets |
| Oxamyl (Vydate) | 0.2 | 0.2 |  | Kidney damage |  | Insecticide on apples, potatoes, tomatoes |
| PAHs (benzo(a)pyrene) | zero | 0.0002 |  | Cancer |  | Coal tar coatings; burning organic matter: volcanoes, fossil fuels |
| PCBs | zero | 0.0005 |  | Cancer |  | Coolant oils from electrical transformers; plasticizers |
| Pentachlorophenol | zero | 0.001 |  | Liver and kidney effects, and cancer |  | Wood preservatives, herbicide, cooling tower wastes |
| Phthalate, (di (2-ethylhexyl) | zero | 0.006 |  | Cancer |  | PVC and other plastics |
| Picloram | 0.5 | 0.5 |  | Kidney, liver damage |  | Herbicide on broadleaf and woody plants |
| Simazine | 0.004 | 0.004 |  | Cancer |  | Herbicide on grass sod, some crops, aquatic algae |
| Styrene | 0.1 | 0.1 |  | Liver, nervous system damage |  | Plastics, rubber, resin, drug industries; leachate from city landfills |
| Tetrachloroethylene | zero | 0.005 |  | Cancer |  | Improper disposal of dry cleaning and other solvents |
| Toluene | 1 | 1 |  | Liver, kidney, nervous, circulatory |  | Gasoline additive; manufacturing and solvent operations |
| Toxaphene | zero | 0.003 |  | Cancer |  | Insecticide on cattle, cotton, soybeans; canceled 1982 |
| 2,4,5- TP | 0.05 | 0.05 |  | Liver and kidney damage |  | Herbicide on crops, right-of-way, golf courses; canceled 1983 |
| 1,2,4- Trichlorobenzene | 0.07 | 0.07 |  | Liver, kidney damage |  | Herbicide production; dye carrier |
| 1,1,2- Trichloroethane | 0.003 | 0.005 |  | Kidney, liver nervous system |  | Solvent in rubber other organic products; chemical production wastes |
| Xylenes (total) | 10 | 10 |  | Liver, kidney, nervous system |  | By-product of gasoline refining; paints, inks, detergents |
| Lead and Copper |  |  |  |  |  |  |
| Lead* | zero | TT+ |  | Kidneys, nervous system damage |  | Natural/industrial deposits; plumbing, solder, brass alloy faucets |
| Copper | 1.3 | TT# |  | Gastrointestinal irritation |  | Natural-industrial deposits; wood preservatives, plumbing |
| Other Interim Standards |  |  |  |  |  |  |
| Beta/photon emitters | zero | 4 mrem/yr |  | Cancer |  | Decay of radionuclides in natural and man-made deposits |
| Alpha emetters | zero | 15 pCi/L |  | Cancer |  | Decay of radionuclides in natural deposits |
| Combined Radium 226/228 | zero | 5 pCi/L |  | Bone Cancer |  | Natural deposits |
| Arsenic* | 0.05 | 0.05 |  | Skin, nervous system toxicity |  | Natural deposits; Smelters, glass, electronics wastes; Orchards |
| Total Trihalomethanes | zero | 0.10 |  | Cancer |  | Drinking water chlorination by-products |
| Notes: *Contaminants with interim standards which have been revised. TT= Special treatment techniques required += Action Level 0.015mg/L #= Action Level 1.3mg/L pCi= piocuries |  |  |  |  |  |  |

NATIONAL PRIMARY DRINKING WATER REGULATIONS:

TURBIDITY

SYSTEM TYPE:  	COMMUNITY, NON-COMMUNITY SURFACE WATER AND GROUND WATER “UNDER THE INFLUENCE” OF SURFACE WATER.

MONITORING:

BASED ON THE REQUIREMENTS OF SURFACE WATER TREATMENT RULE:  ONE

“RAW” WATER SAMPLE PER DAY.  FINISHED WATER SAMPLED ONCE EVERY

FOUR HOURS THAT THE SYSTEM IS IN OPERATION.  FINISHED WATER MAY BE SAMPLED

CONTINUOUSLY OR WITH “GRAB” SAMPLES.  SURFACE SOURCES AND GROUND WATER

SOURCES “UNDER THE INFLUENCE” OF SURFACE WATER MUST SAMPLE AS IF SURFACE

WATER.

MAXIMUM CONTAMINANT LEVEL:  	BASED ON FILTRATION TYPE

| FILTRATION TYPE | TURBIDITY LEVEL |
| --- | --- |
| CONVENTIONAL | 0.5 NTU in 95% of samples (*) |
| DIRECT | 0.5 NTU in 95% of samples (*) |
| DIATOMACEOUS EARTH | 1.0 NTU in 95% of samples |
| SLOW SAND | 1.0 NTU in 95% of samples |

(*) A WAIVER TO 1.0 NTU MAY BE GRANTED BY STATE.

CHECK SAMPLING & PUBLIC NOTIFICATION:

IF A TURBIDITY SAMPLE EXCEEDS 1 NTU, RESAMPLE AS SOON AS PRACTICAL,

PREFERABLY WITHIN 1 HR.  IF REPEAT SAMPLE CONFIRMS THIS LIMIT EXCEEDED,

REPORT TO STATE WITHIN 48 HRS.- USING REPEAT SAMPLES TO CALCULATE AVERAGE.

IF MONTHLY AVERAGE OF 1 NTU OR 2 DAY AVERAGE OF 5 NTU IS EXCEEDED, REPORT

TO STATE WITHIN 48 HRS. AND NOTIFY PUBLIC.

NATIONAL PRIMARY DRINKING WATER REGULATIONS:

RADIOACTIVITY

| CONTAMINANT | SYSTEM TYPE | MAXIMUM CONTAMINANT LEVEL (MG/L) | DRINKING WATER HEALTH EFFECTS |
| --- | --- | --- | --- |
| GROSS ALPHA PARTICLE ACTIVITY (INCLUDING RADUM-226 BUT EXCLUDING RADON AND URANIUM.) | COMM. & NON-TRANS | 15 pCi/L | CANCER |
| COMBINED RADIUM-226 AND RADIUM-228 | “ “ | 5 pCI/L | CANCER |
| MAN-MADE BETA (*) PARTICLES AND PHOTONS EMITTERS | “ “ | 4 MILLIREM PER YEAR | CANCER |

(*)    THE BETA PARTICLE MCL IS BASED ON DOSE EQUIVALENT TO TNTERNAL

ORGANS.  ON AVERAGE THIS 4 MILLIREM MCL WOULD EQUAL 50 pCI/L.

MONITORING:

| SURFACE ONLY OR COMBINATION: AND | COMMUNITY SYSTEM MUST MONITOR FOR GROSS ALPHA. |
| --- | --- |
| GROUND WATER ONLY: | IF GROSS ALPHA DOES NOT EXCEED 5 pCI/L, MONITOR EVERY 4 YEARS BY THE STATE. |

SURFACE SUPPLIES SERVING 100,000 POPULATION MUST MONITOR FOR GROSS BETA, Sr-

90 & H-3.  IF GROSS BETA DOES NOT EXCEED 50 pCI/L, Sr-90 8pCI/L, H-3 20,000 pCi/L, MONITOR EVERY 4 YRS.

ANY COMMUNITY, AS DESIGNATED BY THE STATE, DOWNSTREAM FROM A NUCLEAR

FACILITY MUST MONITORED QUARTERLY FOR GROSS BETA AND I-131, AND ANNUAL

MONITORING FOR Sr-90 AND H-3.

CHECK SAMPLING & PUBLIC NOTIFICATION:

BASED ON ANNUAL AVERAGE OF QUARTERLY SAMPLES, IF MCL IS EXCEEDED, NOTIFY

PUBLIC.  SUBSEQUENT QUARTERLY MONITORING (MONTHLY MONITORING FOR BETA

PARTICLE) TO BE CONTINUED UNTIL PROBLEM IS RECTIFIED.

NATIONAL PRIMARY DRINKING WATER REGULATIONS:

ORGANIC CHEMICALS- TRIHALOMETHANES

| CONTAMINANT | SYSTEM TYPE | MAXIMUM CONTAMINANT LEVEL (MG/L) | DRINKING WATER HEALTH EFFECTS |
| --- | --- | --- | --- |
| TRIHALOMETHANES | COMM. & NON-TRANS | 0.10 | CANCER |

MONITORING:

QUARTERLY SAMPLING OF CHLORINATING COMMUNITY SYSTEMS SERVING AT LEAST

10,000.  IF LEVELS LESS THAN MCL, THEN ON END-OF-LINE SITE, OTHERWISE 4 SAMPLES.

MAXIMUM CONTAMINANT LEVEL:

BASED ON RUNNING QUARTERLY AVERAGE OF SUM OF CHLOROFORM, BROMOFORM,

BROMODICHLOROMETHANE , CHLORODIBROMOMETHANE.

| ORGANIC CHEMICALS- UNREGULATED |  |
| --- | --- |
| SYNTHETIC | VOLATILE |
| ALDRIN, CARBARYL | BROMOBENZENE, BROMODICHLOROMETHANE, BROMOFORM |
| DICAMBA, DIELDRIN | BROMOMETHANE, CHLOROBENZENE, CHLORODIBROMETHANE, |
| METHOMYL, 3-HYDOOX-FURAN, BUTACHLOR, METHACHLOR, METRIBUZIN, PROPACHLOR | CHLOROFORM, CHLORETHANE, CHLOROMETHANE, DICHLOROBENZENE, m-DICHLOROBENZENE, 1,1- DICHLOROETHANE, 1,3-DICHLOROPROPANE, 2,2-DICHLOROPROPANE, 1,3-DICHLOROPROPENE, 1,1-DICHLOROPROPENE, 1, 1, 1, 2-TETRACHLORETHANE, 1, 1, 2, 2-TETRACHLORETHANE, 1, 2, 3-TRICHLOROPROPANE, BROMOCHLOROMETHANE, n-BUTYLBENZENE, TETRA-BUTYLBENZENE, o-CHLOROTOLUENE, p-CHLOROTOLUENE, 1, 2, 4-TRIMETHYLBENZENE, SEC-BUTYLBENZENE, p-ISOPROPYLTOLUENE, n-PROPYLBENZENE, DICHLORODIFLUOROMEHTANE, FLUOROTRICHLOROMETHANE, NAPTHALENE, HEXACHLOROBUTADIENE, 1, 2, 3-TRICHLOROBENZENE, 1, 2, 4-TRICHLORBNEZENE, 1,3,5-TRIMETHLBENZENE, ISOPORPYLBENZENE |

MONITORING:   SAME AS REGULATED VOCs AND SOCs.

MAXIMUM CONTAMINANT LEVEL:  NO MCL HAS BEEN ESTABLISHED.

PUBLIC NOTIFICATION:  NOTIFY PERSONS SERVED BY THE SYSTEM OF THE AVAILABILITY OF THE RESULTS.  CHECK SAMPLING AS SPECIFIED BY STATE.

DRINKING WATER CHEMICAL SAMPLING PROTOCOL

ALL ACTIVE SOURCES OF DRINKING WATER ARE SAMPLED FOR ALL REGULATED AND UNREGULATED CHEMICAL COMPOUNDS REQUIRED UNDER FEDERAL REGULATIONS ON A ONE OR FIVE YEAR CYCLE.  SURFACE WATER SOURCES ARE SAMPLED ANNUALLY AND GROUND WATER SOURCES ON A THREE TO FIVE YEAR CYCLE.  FOR GROUND WATER SYSTEMS THESE SAMPLES ARE COLLECTED AT THE WELLHEAD. SURFACE WATER IS SAMPLED AT THE WATER TREATMENT PLANT AFTER TREATMENT. IF A DETECTION OF A COMPOUND OCCURS, THE SYSTEM IS RE-SAMPLED AT THE WELLHEAD AND FROM THE DISTRIBUTION SYSTEM NEAR THE WELL.  IF THE DETECTION OF A REGULATED COMPOUND IS CONFIRMED IN A GROUND WATER OR SURFACE WATER DISTRIBUTION SYSTEM SAMPLE OR AT THE WELLHEAD, QUARTERLY DISTRIBUTION SYSTEM SAMPLES ARE COLLECTED.  THE SYSTEM IS PUT ON ANNUAL SAMPLING IF THE AVERAGE OF FOUR QUARTERLY SAMPLES REMAINS BELOW THE MAXIMUM CONTAMINANT LEVEL (MCL).  A SYSTEM WITH AN AVERAGE ABOVE THE MCL IS IN VIOLATION OF DRINKING WATER REGULATIONS AND IS REQUIRED TO NOTIFY THEIR CUSTOMERS AND TO UNDERTAKE CORRECTIVE ACTIVITIES.

CRYPTOSPORIDIUM

CRYPTOSPORIDIUM, A PATHOGEN WHICH WILL BE REGULATED UNDER THE ENHANCED SURFACE WATER TREATMENT RULE WILL ALSO BE INCLUDED IN THE INVENTORY OF POTENTIAL SOURCES OF PATHOGENS IN THE WATERSHED.  CRYPTOSPORIDIUM HAS EXTREME RESISTANCE TO MOST DISINFECTANTS, INCLUDING CHLORINE.  THUS, FILTRATION IS REQUIRED TO REMOVE IT.  THE SOURCE OF CRYPOTSPORIDIUM IS LIVESTOCK AND SEWAGE WASTE.